SIGNAL PROCESSING PROGRAMS

1.QRS detection

!pip install numpy scipy matplotlib wfdb

import numpy as np

import matplotlib.pyplot as plt

from scipy import signal

import wfdb

record = wfdb.rdrecord('100', sampto=2000, pn_dir='mitdb')

ecg = record.p_signal[:, 0]

fs = record.fs # Sampling frequency

f_bandpass = [0.5, 40] # Frequency range (Hz)

order_bandpass = 2

b_bandpass, a_bandpass = signal.butter(order_bandpass,

[f_bandpass[0] / (fs / 2), f_bandpass[1] / (fs / 2)],

btype='bandpass')

ecg_filtered = signal.filtfilt(b_bandpass, a_bandpass, ecg)

peaks, _ = signal.find_peaks(ecg_filtered, height=0.5, distance=round(0.6* fs))

RR_intervals = np.diff(peaks) / fs

heart_rate = 60 / np.mean(RR_intervals)

QRS_window_pre = round(0.06 * fs)

QRS_window_post = round(0.06 * fs)

plt.figure(figsize=(12,6))

plt.plot(ecg_filtered, label='Filtered ECG', color='b')

plt.plot(peaks, ecg_filtered[peaks], 'ro', label='Detected R-peaks')

plt.title(f'QRS Detection (Estimated Heart Rate: {heart_rate:.2f} bpm)')

plt.xlabel('Samples')

plt.ylabel('Amplitude (mV)')

plt.legend()

plt.grid(True)   plt.show()

2. EEG classification

!pip install mne

import matplotlib.pyplot as plt

import numpy as np

from scipy.signal import butter , filtfilt

import mne

file_path="/content/S002R02.edf"

raw=mne.io.read_raw_edf(file_path,preload=True)

data_fp1=raw.get_data(picks=['Fp1.']).flatten()

fs=int(raw.info['sfreq'])

t=np.arange(len(data_fp1))/fs

order=4

def bandpass_filter(data,lowcut,highcut,fs,order=4):

myquist=0.5*fs

low=lowcut/myquist

high=highcut/myquist

b,a=butter(order,[low,high],btype='band')

return filtfilt(b,a,data)

delta_wave=bandpass_filter(data_fp1,0.5,4,fs,order)

theta_wave=bandpass_filter(data_fp1,4,8,fs,order)

alpha_wave=bandpass_filter(data_fp1,8,12,fs,order)

beta_wave=bandpass_filter(data_fp1,12,30,fs,order)

plt.figure()

plt.subplot(4,1,1)

plt.plot(t,delta_wave)

plt.title('Delta wave (0.5-4 Hz)')

plt.subplot(4,1,2)

plt.plot(t,theta_wave)

plt.title('Theta wave (4-8 Hz)')

plt.subplot(4,1,3)

plt.plot(t,alpha_wave)

plt.title('Alpha wave (8-12 Hz)')

plt.subplot(4,1,4)

plt.plot(t,beta_wave)

plt.title('Beta wave (12-30 Hz)')

plt.xlabel('Time(s)')

plt.ylabel('Amplitude')

3. Power Spectrum

!pip install mne numpy matplotlib scipy

import numpy as np

import matplotlib.pyplot as plt

from scipy.signal import welch

import mne

file_path="/content/S002R02.edf"

raw=mne.io.read_raw_edf(file_path,preload=True)

fs=int(raw.info['sfreq'])

data,times=raw.get_data(picks=['Fp1.'],return_times=True)

dataFp1_array=data.flatten()

f_welch,pxx=welch(dataFp1_array,fs=fs,nperseg=1024)

plt.figure()

plt.plot(f_welch,10*np.log10(pxx))

plt.title('Power Spectral Density of EEG Signal (Fp1)')

plt.xlabel('Frequency(Hz)')

plt.ylabel('Power/Frequency(dB/Hz)')

plt.grid()

plt.show()

4.Statistical Parameters

import scipy.io

from scipy.signal import butter, filtfilt

import numpy as np

file_paths=['/content/115m (15).mat','/content/202m (15).mat','/content/223m (5).mat']

fc=0.5

fs = 1000

for i,file_path in enumerate(file_paths,start=1):

data=scipy.io.loadmat(file_path)

ecg_signal=data['val'].flatten()

b,a=butter(4,fc/(fs/2),btype='high')

ecg_filtered=filtfilt(b,a,ecg_signal)

manual_mean_original = np.sum(ecg_signal) / len(ecg_signal)

mean_original=np.mean(ecg_signal)

std_original=np.std(ecg_signal)

var_original=np.var(ecg_signal)

manual_mean_filtered = np.sum(ecg_filtered) / len(ecg_filtered)

mean_filtered=np.mean(ecg_filtered)

std_filtered=np.std(ecg_filtered)

var_filtered=np.var(ecg_filtered)

print(f'Original Signal {i}')

print(f' Mean: {mean_original:.2f}')

print(f'Mean (manual): {manual_mean_original:.2f}')

print(f' Standard Deviation: {std_original:.2f}')

print(f' Variance: {var_original:.2f}')

print(f'Filtered Signal {i}')

print(f' Mean: {mean_filtered:.2f}')

print(f'Mean (manual): {manual_mean_filtered:.2f}')

print(f' Standard Deviation: {std_filtered:.2f}')

print(f' Variance: {var_filtered:.2f}')

IMAGE PROCESSING PROGRAMS

1.Boundary detection

import cv2

import numpy as np

import matplotlib.pyplot as plt

from skimage import measure

from skimage.morphology import disk, opening, closing

from skimage.filters import threshold_otsu

image_path = '/content/MRI.jpg'

img = cv2.imread(image_path, cv2.IMREAD_GRAYSCALE)

thresh = threshold_otsu(img)

binary_img = img > thresh

binary_img = opening(binary_img, disk(5)) # Remove small objects

binary_img = closing(binary_img, disk(15)) # Close small holes

contours = measure.find_contours(binary_img, 0.8)

fig, ax = plt.subplots()

ax.imshow(img, cmap='gray')

for contour in contours:

ax.plot(contour[:, 1], contour[:, 0], color='yellow', linewidth=2)

for contour in contours:

ax.fill(contour[:, 1], contour[:, 0], 'w', alpha=0.3)

ax.set_title('Brain Boundary Marked')

plt.axis('off')

plt.show()

2.Image fusion using haar transform

!pip install pywt scikit-image opencv-python

import numpy as np

import cv2

import pywt

import matplotlib.pyplot as plt

from skimage.metrics import peak_signal_noise_ratio as psnr

i1 = cv2.imread('/content/CT.jpg', cv2.IMREAD_GRAYSCALE)

i2 = cv2.imread('/content/MRI.jpg', cv2.IMREAD_GRAYSCALE)

if i1.shape != i2.shape:

i2 = cv2.resize(i2, (i1.shape[1], i1.shape[0]))

plt.figure(figsize=(12, 6))

plt.subplot(1,3,1)

plt.imshow(i1, cmap='gray')

plt.title('Source Image 1')

plt.subplot(1,3,2)

plt.imshow(i2, cmap='gray')

plt.title('Source Image 2')

cA1, (cH1, cV1, cD1) = pywt.dwt2(i1, 'haar')

cA2, (cH2, cV2, cD2) = pywt.dwt2(i2, 'haar')

cA = np.maximum(cA1, cA2)

cH = np.maximum(cH1, cH2)

cV = np.maximum(cV1, cV2)

cD = np.maximum(cD1, cD2)

fused_image = pywt.idwt2((cA, (cH, cV, cD)), 'haar')

fused_image = np.clip(fused_image, 0, 255)

fused_image_uint8 = fused_image.astype(np.uint8)

plt.subplot(1,3,3)

plt.imshow(fused_image_uint8, cmap='gray')

plt.title('DWT Max Fusion Rule')

plt.show()

h, w = i1.shape

fused_image_uint8_cropped = fused_image_uint8[:h, :w]

psnr_1 = psnr(i1, fused_image_uint8_cropped)

psnr_2 = psnr(i2, fused_image_uint8_cropped)

print(f'PSNR with respect to Source Image 1: {psnr_1:.4f}')

print(f'PSNR with respect to Source Image 2: {psnr_2:.4f}')

3. Image fusion without using haar transform

!pip install pywavelets matplotlib opencv-python

import cv2

import pywt

import matplotlib.pyplot as plt

img=cv2.imread('/content/MRI.jpg',cv2.IMREAD_GRAYSCALE)

cA,(cH,cV,cD)=pywt.dwt2(img,'haar')

plt.figure(figsize=(10,8))

plt.subplot(2,2,1)

plt.imshow(cA,cmap='gray')

plt.title('Approximation Coefficients')

plt.axis('off')

plt.subplot(2,2,2)

plt.imshow(cH,cmap='gray')

plt.title('Horizontal Detail Coefficients')

plt.axis('off')

plt.subplot(2,2,3)

plt.imshow(cV,cmap='gray')

plt.title('Vertical Detail Coefficients')

plt.axis('off')

plt.subplot(2,2,4)

plt.imshow(cD,cmap='gray')

plt.title('Diagonal Detail Coefficients')

plt.axis('off')

plt.show()

reconstructed=pywt.idwt2((cA,(cH,cV,cD)),'haar')

plt.figure(figsize=(10,4))

plt.subplot(1,2,1)

plt.imshow(img,cmap='gray')

plt.title('Original Image')

plt.axis('off')

plt.subplot(1,2,2)

plt.imshow(reconstructed,cmap='gray')

plt.title('Reconstructed Image')

plt.axis('off')

plt.show()

4.Wavelet on ECG

!pip install mne

import numpy as np

import matplotlib.pyplot as plt

import pywt

import scipy.io

import mne

data = scipy.io.loadmat('/content/115m (15).mat')

y = data['val'].flatten()

l1 = h1 = None

l2 = h2 = None

l3 = h3 = None

yy = y.copy()

for i in range(1, 4):

CA, CD = pywt.dwt(yy, 'db1', mode='sym')

if i == 1:

l1, h1 = CA, CD

elif i == 2:

l2, h2 = CA, CD

else:

l3, h3 = CA, CD

yy = CA

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(CA)

plt.title(f'Approximation Coefficients at Level {i}')

plt.subplot(2, 1, 2)

plt.plot(CD, 'r')

plt.title(f'Detail Coefficients at Level {i}')

plt.tight_layout()

plt.show()

thres = 1.5

h1[np.abs(h1) < thres] = 0

h2[np.abs(h2) < thres] = 0

h3[np.abs(h3) < thres] = 0

ll2 = pywt.idwt(l3, h3, 'db1', mode='sym')

ll1 = pywt.idwt(ll2, h2, 'db1', mode='sym')

xx = pywt.idwt(ll1, h1, 'db1', mode='sym')

plt.figure(figsize=(10, 6))

plt.plot(y, 'b', label='Original (Noisy)')

plt.plot(xx, 'r', label='Denoised')

plt.title('ECG Signal (Original vs Denoised)')

plt.legend()

plt.grid(True)

plt.show()

diff = y - xx

MSE = np.mean(diff ** 2)

print(f'MSE (ECG): {MSE:.4f}')

5.Wavelet on EEG

import numpy as np

import matplotlib.pyplot as plt

import pywt

import scipy.io

import mne

raw = mne.io.read_raw_edf('/content/S002R02.edf', preload=True)

eeg_signal = raw.get_data(picks=[0])[0]

l1 = h1 = None

l2 = h2 = None

l3 = h3 = None

yy = eeg_signal.copy()

for i in range(1, 4):

CA, CD = pywt.dwt(yy, 'db1', mode='sym')

if i == 1:

l1, h1 = CA, CD

elif i == 2:

l2, h2 = CA, CD

else:

l3, h3 = CA, CD

yy = CA

plt.figure(figsize=(10, 6))

plt.subplot(2, 1, 1)

plt.plot(CA)

plt.title(f'Approximation Coefficients at Level {i}')

plt.subplot(2, 1, 2)

plt.plot(CD, 'r')

plt.title(f'Detail Coefficients at Level {i}')

plt.tight_layout()

plt.show()

# Thresholding

thres = 1.5

h1[np.abs(h1) < thres] = 0

h2[np.abs(h2) < thres] = 0

h3[np.abs(h3) < thres] = 0

ll2 = pywt.idwt(l3, h3, 'db1', mode='sym')

ll1 = pywt.idwt(ll2, h2, 'db1', mode='sym')

xx = pywt.idwt(ll1, h1, 'db1', mode='sym')

# Plot Original vs Denoised

plt.figure(figsize=(10, 6))

plt.plot(eeg_signal, 'b', label='Original (Noisy)')

plt.plot(xx, 'r', label='Denoised')

plt.legend()

plt.title('EEG Signal (Original vs Denoised)')

plt.grid(True)

plt.show()

MSE = np.mean((eeg_signal - xx)**2)

print(f'MSE (EEG): {MSE:.4f}')